| Date Issued: | Valid Until: |
| --- | --- |
| Location of Work: |  |
| Type of Work: |  |
| Physical Facility Phone Number: |  |
| Hot Work Operator (Name and Title): |  |
| Type of Welding Equipment: |  |
| Required PPE: |  |

FIRE SAFETY PRECAUTIONS - Please mark each box as items are completed.

BEFORE THE WORK - All of the following precautions must be taken:

Cutting and/or welding equipment must be thoroughly inspected and found to be in good repair, free of damage or defects.

Hot work operator equipped with communication device, protective equipment and is knowledgeable regarding the emergency response procedures for the facility.

Floor areas under and at least 35 feet around the cutting/welding operation must be swept clean of combustible and flammable materials.

Flammable and combustible liquids must be relocated at least 35 feet away from the cutting/welding operation.

Ensure area is cordoned off to prevent unauthorized access during hot work activities.

Fire watch person(s) are to be supplied with multi purpose dry chemical, portable fire extinguisher such that it is immediately available for use and trained in its use.

Where applicable, the following precautions will also be taken before the work begins:

Fire resistant shields must cover combustible floors and must be below any elevated cutting/welding operation.

All floor and wall openings must be covered to prevent sparks/slag from traveling to other unprotected areas.

Containers in or on which cutting/welding will take place must be purged of flammable vapors.

Fire alarm system deactivation

Fume collector or local exhaust ventilation in work area to remove smoke/vapor.

Protective screens positioned to protect nearby personnel from hazardous light.

DURING / AFTER THE WORK - The following precautions will be taken:

Fire watch must inspect the area for at least 30 minutes after all cutting/welding ceases.

____________________________________________________________________________________

This permit has been reviewed with and understood by the hot work operator before the commencement of work activities.

| Physical Facility Supervisor Signature Date |  |
| --- | --- |
| Hot Work Operator Signature Date |  |

The work area and all adjacent areas to which sparks and heat might have spread (including floor levels above and below and on opposite side of walls) were inspected 30 minutes after the cutting and or welding operations ceased for the day and were found to be fire safe.

| Hot Work Operator Signature | Date |
| --- | --- |